# Required Components

|  | BILL OF MATERIALS 3D printed switch top 3D printed switch bottom 3.5 mm mono cable 12mm tactile switch 1 4 3 2 |
| --- | --- |

# Required Tools

- Soldering Iron and Solder

- Wire Strippers / Wire Cutters

- Hot Glue Gun and Glue

# Required Personal Protective Equipment (PPE)

- Safety Glasses

# Assembly Instructions

| Cut off the two leads on one side flush. 01. | Bend the two remaining leads to make a 90 angle with the switch. 02. |
| --- | --- |
| Cut off the two plastic mounting lugs completely flush. 03. | Cut the mono jack cable to the desired length. 04. |
| Pull mono cable through the hole in the base of the switch. 05. | Use the wire strippers to strip 2cm. 06. |
| Use wire strippers to strip 1.5cm off the individual wires. 07. | Hold the wire behind the switch and wrap wires around the switch leads. 08. |
| Hold the switch in place and bend the wire in between the two switch leads. 09. | IMPORTANT: After soldering the two leads indicated in the white rectangle, test the switch by plugging it into a switch tester or assistive device. 10. |
| Move the 3D printed base close to the switch and fit the cable in the 3d printed groove. 11. | Add a small drop of hot glue. Do not add too much. Press down on the switch ensuring that it is sitting level. 12. |
| Bury the soldered joint and cable in hot glue. 13. | Place the 3D printed top over the 3D printed base. Align the slot in the top with the joint on the bottom. 14. |
| Apply pressure at one end of the top and snap the top onto the base. Then apply pressure to the other end. 15. | Move the top back and forth to insure it is fully snapped on and can rotate freely without friction. 16. |